A P P E N D I X   A

Updated CEIDARS Table with PM2.5 Fractions

Table A - Updated CEIDARS Table with PM2.5 Fractions

| SCC MAIN CATEGORY | SCC SUBCATEGORY | PM2.5 Fraction of Total PM | PM10 Fraction of Total PM | PM2.5 Fraction of PM10 |
| --- | --- | --- | --- | --- |
| ASBESTOS REMOVAL |  | 0.500 | 0.500 | 1.000 |
| ASPHALT PAVING / ROOFING | FUGITIVE EMISSIONS | 0.925 | 0.960 | 0.964 |
|  | MANUFACTURING | 0.945 | 0.980 | 0.964 |
| BURNING | AGRICULTURE/FIELD CROPS, WEED ABATEMENT | 0.938 | 0.984 | 0.954 |
|  | FOREST MANAGEMENT, TIMBER AND BRUSH FIRE | 0.854 | 0.961 | 0.889 |
|  | ORCHARD PRUNINGS | 0.925 | 0.981 | 0.943 |
|  | RANGE MANAGEMENT, WASTE BURNING | 0.932 | 0.983 | 0.948 |
|  | UNPLANNED STRUCTURAL FIRES | 0.914 | 0.980 | 0.933 |
| CEMENT MANUFACTURING |  | 0.620 | 0.920 | 0.674 |
| CHEMICAL MANUFACTURING | FERTILIZER-UREA | 0.950 | 0.960 | 0.990 |
|  | ORGANIC AND INORGANIC CHEMICALS | 0.890 | 0.900 | 0.989 |
| COATINGS, SOLVENTS, INKS AND DYES | SOLVENT BASED | 0.925 | 0.960 | 0.964 |
|  | WATER-BASED COATING | 0.620 | 0.680 | 0.912 |
| CONSUMER PRODUCTS |  | 0.925 | 0.960 | 0.964 |
| COOKING | BAKING, CHARBROILING, DEEP FAT FRYING | 0.420 | 0.700 | 0.600 |
| COOLING TOWER |  | 0.420 | 0.700 | 0.600 |
| DRY CLEANING |  | 0.925 | 0.960 | 0.964 |
| ELECTROPLATING | HEXAVALENT CHROME, CADMIUM | 1.000 | 1.000 | 1.000 |
|  | ZINC AND COPPER | 0.925 | 0.960 | 0.964 |
| EXTERNAL COMBUSTION | COAL, COKE, LIGNITE | 0.150 | 0.400 | 0.375 |
|  | GASEOUS FUEL-EXCEPT PETROLEUM AND INDUSTRIAL PROCESS HEATERS | 1.000 | 1.000 | 1.000 |
|  | GASEOUS FUEL-PETROLEUM AND INDUSTRIAL PROCESS HEATER ONLY | 0.930 | 0.950 | 0.979 |
|  | LIQUID FUEL-EXCEPT RESIDUAL OIL | 0.967 | 0.976 | 0.991 |
|  | RESIDUAL OIL-EXCEPT UTILITY BOILERS | 0.760 | 0.870 | 0.874 |
|  | RESIDUAL OIL-UTILITY BOILERS ONLY | 0.953 | 0.970 | 0.982 |
|  | STEEL FURNACE | 0.930 | 0.980 | 0.949 |
|  | WOOD/BARK WASTE | 0.927 | 0.997 | 0.930 |
| FABRICATED METALS | ABRASIVE BLASTING | 0.790 | 0.860 | 0.919 |
|  | ARC WELDING, OXY FUEL, COPPER, ZINC, BATH | 0.925 | 0.960 | 0.964 |
| FOOD AND AGRICULTURE | COFFEE ROASTING | 0.610 | 0.620 | 0.984 |
|  | FERMENTATION, RENDERING, FISH AND NUT PROCESSING | 0.420 | 0.700 | 0.600 |
|  | GRAIN ELEVATORS | 0.010 | 0.290 | 0.034 |
|  | GRAIN MILLING, DRYING | 0.400 | 0.540 | 0.741 |
|  | LIVESTOCK WASTE | 0.420 | 0.700 | 0.600 |
| FUGITIVE DUST | AGRICULTURAL TILLING DUST | 0.101 | 0.454 | 0.222 |
|  | CONSTRUCTION AND DEMOLITION | 0.102 | 0.489 | 0.208 |
|  | LANDFILL DUST | 0.102 | 0.489 | 0.208 |
|  | LIVESTOCK DUST | 0.055 | 0.482 | 0.114 |
|  | PAVED ROAD DUST | 0.077 | 0.457 | 0.169 |
|  | UNPAVED ROAD DUST | 0.126 | 0.594 | 0.212 |
| FUGITIVE EMISSIONS - ORGANIC AND INORGANIC | LIQUID FUEL STORAGE/HANDLING, LOADING, UNLOADING DISPENSING | 0.925 | 0.960 | 0.964 |
|  | NATURAL GAS PRODUCTION, CRUDE OIL PRODUCTION, PETROLEUM REFINING | 0.555 | 0.610 | 0.910 |
|  | ORGANIC AND INORGANIC CHEMCALS | 0.925 | 0.960 | 0.964 |
|  | PROCESSING | 0.925 | 0.960 | 0.964 |
|  | WELL CELLEARS, PUMPS, VALVES, FLAGES, SEALS | 0.925 | 0.960 | 0.964 |

Table A - Updated CEIDARS Table with PM2.5 Fractions (Continued)

| SCC MAIN CATAGORY | SCC SUBCATAGORY | PM2.5 Fraction of Total PM | PM10 Fraction of Total PM | PM2.5 Fraction of PM10 |
| --- | --- | --- | --- | --- |
| HEALTH CARE, LABS | STERILIZATION | 0.420 | 0.700 | 0.600 |
| INCINERATOR, AFTERBURNER, FLARES | GASEOUS FUEL | 1.000 | 1.000 | 1.000 |
|  | LIQUID FUEL | 0.967 | 0.976 | 0.991 |
|  | SOLID FUEL | 0.200 | 0.300 | 0.667 |
| INTERNAL COMBUSTION | DISTILLATE AND DIESEL-ELECTRIC GENERATION | 0.937 | 0.960 | 0.976 |
|  | DISTILLATE AND DIESEL-EXCEPT ELECTRIC GENERATION | 0.967 | 0.976 | 0.991 |
|  | GASEOUS FUEL | 0.992 | 0.994 | 0.998 |
|  | GASOLINE | 0.992 | 0.994 | 0.998 |
|  | JET FUEL | 0.967 | 0.976 | 0.991 |
|  | SOLID PROPELLANT | 0.927 | 0.997 | 0.930 |
| MINERAL PROCESS LOSS | BRICK, CEMENT, FIBERGLASS, GLASS MFG. | 0.146 | 0.500 | 0.292 |
|  | COAL CLEANING, SURFACE COAL MINE, NONMETALLIC MINERAL | 0.146 | 0.500 | 0.292 |
|  | GRINDING, CRUSHING, SURFACE BLASTING | 0.146 | 0.500 | 0.292 |
|  | LOADING AND UNLOADING BULK MATERIALS | 0.146 | 0.500 | 0.292 |
| MINERAL PRODUCTS | CLAY AND RELATED PRODUCTS GRINDING OPERATIONS | 0.513 | 0.560 | 0.916 |
|  | CRUSHING, SCREENING, BLASTING, LOADING AND UNLOADING | 0.030 | 0.100 | 0.300 |
|  | FIBERGLASS MANUFACTURING | 0.992 | 0.994 | 0.998 |
|  | GLASS MELTING FURNACE | 0.963 | 0.980 | 0.983 |
|  | GYPSUM MANUFACTURING | 0.495 | 0.880 | 0.563 |
|  | LIME MANUFACTURING | 0.117 | 0.300 | 0.390 |
|  | STONE QUARRYING | 0.146 | 0.500 | 0.292 |
| OFF-ROAD EQUIPMENT | DIESEL | 0.920 | 1.000 | 0.920 |
|  | GASEOUS FUEL | 0.992 | 0.994 | 0.998 |
|  | GASOLINE | 0.680 | 0.900 | 0.756 |
| ON-ROAD VEHICLES | BRAKE WEAR | 0.420 | 0.980 | 0.429 |
|  | DIESEL | 0.920 | 1.000 | 0.920 |
|  | GASOLINE-CATALYST | 0.900 | 0.970 | 0.928 |
|  | GASOLINE-NO CATALYST | 0.680 | 0.900 | 0.756 |
|  | HEAVY, MEDIUM, LIGHT DUTY TRUCKS AND VEHICLES, MOTORHOMES, BUSES, MOTORCYCLES | 0.925 | 0.960 | 0.964 |
|  | TIRE WEAR | 0.250 | 1.000 | 0.250 |
| PETROLEUM INDRY | ASPHALT CONCRETE | 0.333 | 0.400 | 0.833 |
| PRIMARY AND SECONDARY METALS | ELECTRO REDUCTION, FURNACE, FLUXING, STORAGE, PROCESSING | 0.903 | 0.950 | 0.951 |
|  | IRON & STEEL, FOUNDARY, HEAT TREATING | 0.860 | 0.960 | 0.896 |
|  | STEEL FURNACE | 0.600 | 0.830 | 0.723 |
| RESIDENTIAL FIREPLACES AND WOOD COMBUSTION |  | 0.900 | 0.935 | 0.963 |
| SHIPS | DIESEL | 0.920 | 1.000 | 0.920 |
|  | LIQUID FUEL | 0.937 | 0.960 | 0.976 |
| TRAINS | HAULING, SWITCHING | 0.920 | 1.000 | 0.920 |
| WASTEWATER, SEWAGE TREATMENT, DIGESTER |  | 0.925 | 0.960 | 0.964 |
| WOOD PRODUCTS | SANDING | 0.885 | 0.920 | 0.962 |
|  | SAWING | 0.283 | 0.400 | 0.708 |